PRM Simulation Benchmark

User Guide & Developer Reference

PyBullet-based testing framework for Probabilistic Roadmap algorithms

# 1  Quick Start

## 1.1  Install pybullet

pip install pybullet

# 2  Command Reference

## 2.1  run_single.py

Runs one scenario interactively with full visualisation.

python run_single.py --env ENV --diff D --planner NAME [--gui] [--seed S]

| Flag | Type | Default | Description |
| --- | --- | --- | --- |
| --env | string | simple | Environment: simple, office, or cityscape |
| --diff | int | 0 | Difficulty level: 0, 1, 2, or 3 |
| --planner | string | Basic | Planner algorithm name (must be in PLANNER_CLASSES) |
| --gui | flag | off | Open PyBullet 3D viewer |
| --seed | int | 42 | Random seed for reproducibility |

## 2.2  benchmark.py

Runs cold-start trials across multiple environments, difficulties, and planners. Each trial builds a fresh roadmap from scratch, finds a path, and executes it. Results are aggregated and saved to CSV.

python benchmark.py [--env ...] [--diff ...] [--planners ...] [--trials N] [--csv PATH]

| Flag | Type | Default | Description |
| --- | --- | --- | --- |
| --env | string(s) | all three | One or more of: simple, office, cityscape |
| --diff | int(s) | 0 3 | One or more of: 0, 1, 2, 3 |
| --planners | string(s) | all registered | One or more planner names |
| --trials | int | 50 | Number of trials per (env, diff, planner) cell |
| --gui | flag | off | PyBullet GUI (useful for single-trial debugging) |
| --csv | string | benchmark_results.csv | Output CSV file path |

Example commands

# Full benchmark: Basic planner, static vs hardest, all environments

python benchmark.py --diff 0 3 --trials 50

# Quick test on office only

python benchmark.py --env office --diff 2 --planners Basic --trials 10

# Compare two custom planners

python benchmark.py --planners Basic MyAlgo --trials 50 --csv comparison.csv

# 3  Environments

## 3.1  simple

A 12 × 12 m flat arena with six static obstacles: a horizontal wall across the middle, a vertical wall on the right, a square pillar, an L-shaped barrier, and a small block near the goal. Start is at (1, 1), goal at (11, 11). Default PRM uses 300 samples with dmax = 3.0 m.

## 3.2  office

A 20 × 15 m office floor plan with outer walls, three vertical/horizontal room partitions, and narrow 0.9 m doorways connecting rooms. Four furniture blocks add clutter. Start is at (1.5, 1.5), goal at (18.5, 13.5). Default PRM uses 600 samples with dmax = 3.0 m. This environment is specifically designed to stress-test narrow-passage sampling.

## 3.3  cityscape

A 40 × 40 × 25 m 3D urban environment with 15 buildings ranging from 8 to 24 m tall. The planner works in full (x, y, z) space, representing drone navigation. Start is at (2, 2, 5), goal at (38, 38, 15). Default PRM uses 800 samples with dmax = 8.0 m.

# 4  Difficulty Levels

Each environment supports four difficulty settings that control motion noise and dynamic obstacles.

| Level | Name | Motion Noise | Dynamic Obstacles |
| --- | --- | --- | --- |
| 0 | static | None | None |
| 1 | noise | Gaussian perturbation on each step | None |
| 2 | dynamic | None | Moving obstacles active |
| 3 | noise+dynamic | Gaussian perturbation | Moving obstacles active |

Motion noise details

At each execution step, the commanded position receives additive Gaussian noise. The standard deviation varies by environment: 0.08 m (simple), 0.06 m (office), and 0.15 m (cityscape). Positions are clamped to workspace bounds after noise is applied.

Dynamic obstacle types

| Type | Behaviour | Used In |
| --- | --- | --- |
| PatrolObstacle | Linear back-and-forth between two endpoints at constant speed | All three environments |
| OscillateObstacle | Sinusoidal motion along one axis (simulates sliding doors, barriers) | Simple, Office |
| RandomWalkObstacle | Bounded Gaussian random walk (simulates pedestrians, drones) | Office, Cityscape |

On difficulty 2 and 3, the executor checks upcoming path edges each tick. When a dynamic obstacle blocks the path, it calls the planner’s replan() method from the robot’s current position.

# 5  Benchmark Metrics

The benchmark records five metrics per trial, then aggregates mean and standard deviation across all trials in each (environment, difficulty, planner) cell.

| Metric | Definition | Better |
| --- | --- | --- |
| Success Rate | Fraction of trials where the robot reached the goal without collision | Higher |
| Smoothness | Sum of angular deflections (radians) between consecutive path segments | Lower |
| Min Clearance | Minimum distance (m) from the robot centre to any obstacle surface during execution | Higher |
| Mission Time | Total wall-clock seconds: roadmap construction + path search + execution | Lower |
| Replans | Number of times the executor triggered replanning due to blocked path | Lower |

Results are printed as a formatted table and saved to CSV. Each row in the CSV corresponds to one (env, difficulty, planner) combination with mean and standard deviation for every metric.

# 6  Adding Your Own Algorithm

The framework is designed so you only need to write your planning logic. All collision checking, sampling, graph management, execution, metrics, and visualisation are handled for you.

## 6.1  Step 1 — Copy the template

cp prm_template.py prm_myalgo.py

## 6.2  Step 2 — Implement your class

Your planner must subclass PRMBase and implement two methods: construct() and find(). Optionally override replan() for dynamic environments.

construct(self, N, dmax, verbose=False)

Build the roadmap. N is the number of samples, dmax is the connection radius. Add nodes and edges to self.G (a NetworkX graph) however you want. Return self.G when done.

find(self, start, goal, dmax)

Connect start and goal into the roadmap, search for a path, and return (path, cost) as a tuple. If no path exists, return (None, float(‘inf’)). The path is a list of configuration tuples.

replan(self, current, goal, dmax) — optional

Called by the executor when a dynamic obstacle blocks the current path. The default implementation just calls find() again. Override this if your algorithm can do something smarter, like removing edges near the obstacle and adding local repair samples.

## 6.3  Available utilities from PRMBase

You inherit these and never need to call PyBullet directly:

| Method / Attribute | Returns | Description |
| --- | --- | --- |
| self.G | nx.Graph | The roadmap graph. Add nodes and weighted edges freely. |
| self.bounds | list of (lo, hi) | Workspace limits per axis. |
| self.dim | int | 2 for ground-plane, 3 for aerial. |
| self.radius | float | Robot collision sphere radius. |
| self.obstacle_ids | list of int | PyBullet body IDs of all current obstacles. |
| self.is_free(pt) | bool | Is this configuration collision-free? |
| self.edge_free(v1, v2) | bool | Is the straight line v1 → v2 collision-free? |
| self.clearance(pt) | float | Min distance from robot at pt to nearest obstacle. |
| self.random_sample() | tuple | Rejection-sample a random free configuration. |
| self.dist(a, b) | float | Euclidean distance between two configurations. |
| self.reset_graph() | None | Clear self.G to start fresh. |

## 6.4  Step 3 — Register your planner

Open run_single.py and benchmark.py. Add your import and put it in PLANNER_CLASSES:

from prm_myalgo import MyAlgoPRM

PLANNER_CLASSES = {

"Basic":  BasicPRM,

"MyAlgo": MyAlgoPRM,

}

## 6.5  Step 4 — Run and benchmark

# Interactive test

python run_single.py --env office --diff 0 --planner MyAlgo

# Head-to-head benchmark

python benchmark.py --planners Basic MyAlgo --diff 0 3 --trials 50

# 7  File Reference

| File | Purpose |
| --- | --- |
| prm_base.py | Base class with PyBullet collision checking, sampling, and graph management. All planners inherit from this. |
| prm_basic.py | Standard PRM implementation (baseline). Mirrors the original occupancy-grid algorithm. |
| prm_template.py | Annotated template for writing your own planner. Copy and fill in construct() and find(). |
| environments.py | Builds all three PyBullet environments with configurable difficulty. Returns obstacle IDs, bounds, and plotting data. |
| dynamics.py | Dynamic obstacle controllers (Patrol, Oscillate, RandomWalk) and the Gaussian motion noise model. |
| executor.py | Walks the planned path step-by-step with noise, dynamic obstacle updates, collision detection, and replanning. |
| metrics.py | Computes per-trial metrics (smoothness, clearance, mission time) and aggregates across trials. |
| benchmark.py | Cold-start benchmark runner. Iterates over envs, difficulties, planners, and trials. Outputs table + CSV. |
| run_single.py | Interactive single-run script with matplotlib and optional PyBullet GUI visualisation. |

# 8  Tips

- Start with --diff 0 to verify your algorithm produces valid paths on all three environments before adding noise and dynamic obstacles.

- The office scenario is the hardest for uniform sampling due to narrow doorways. Consider biased sampling near obstacle boundaries.

- Use --trials 10 for quick iteration, then scale to 50 for publishable results.

- The cityscape environment works in 3D — your construct() and find() code works identically because PRMBase handles the dimensionality transparently.

- If success rates are low, increase N (sample count) or dmax (connection radius) in your planner or in the environment defaults in environments.py.

- For dynamic environments (diff 2 and 3), the key differentiator is replan(). A planner that can surgically repair the roadmap rather than rebuilding from scratch will have much lower mission times.